# CYBER-HARDENED BATTERY MANAGEMENT SYSTEM FOR ELECTRIC VEHICLES

AI-Assisted State Estimation with Real-Time CAN-Bus Intrusion Detection

Complete Project Manual — Concept to Simulation to Hardware to Paper to Patent

B.Tech EEE Mini Project · 2nd Year · Galgotias College of Engineering & Technology
12-Week Roadmap · Greater Noida

Team of 5 · Hardware + Simulation · IEEE Conference Paper · Provisional Patent

# Chapter 1 — Executive Summary

This manual is the complete, start-to-finish build guide for a Cyber-Hardened Battery Management System (BMS) mini-project. The project secures Electric Vehicle (EV) battery telemetry against Controller Area Network (CAN) bus cyberattacks — Denial of Service (DoS) flooding, message spoofing, and replay attacks — by combining an on-device Machine Learning (ML) Intrusion Detection System (IDS) with an Extended Kalman Filter (EKF) used for battery State of Charge (SoC) estimation.

The novel, patentable contribution is a feedback loop: when the IDS flags suspicious CAN traffic, its anomaly score dynamically inflates the EKF's measurement-noise covariance (R), causing the filter to automatically down-weight untrusted sensor data and rely on its own internal prediction model instead. The battery keeps reporting an accurate SoC even while the CAN bus is under active attack.

The system runs on two ESP32 WROOM-32 boards — one acting as the BMS master (running the AFE readout, EKF, balancing, and display logic on one core and the IDS on the other), and a second acting as a controlled attacker node used to generate DoS, spoofing, and replay traffic for testing and for the dataset used to train the classifier.

# Chapter 2 — Introduction & Motivation

India's EV market was valued at roughly USD 8.49 billion in 2024 and is projected to grow at a 40.7% CAGR, with estimates placing it near USD 54 billion by 2025. Government schemes such as FAME II and PLI accelerate vehicle deployment faster than electronics security matures.

The Controller Area Network (CAN) bus has no native message authentication. Any device electrically connected to the bus can transmit a frame claiming to be from any other device. A 2025 study demonstrated a crafted sequence of CAN messages could trigger a buffer-overflow disabling pack thermal protection. This vulnerability directly motivates this cyber-hardened BMS design.

# Chapter 3 — Literature Review

Prior literature falls into three distinct categories: EKF-based state estimation (Taborelli & Onori 2014, ICAEEE 2024), CAN-bus intrusion detection (Fakhfakh et al. 2022, Perakovic et al. 2023, Kumar & Singh 2024, Nguyen et al. 2023, Seo et al. 2018), and adaptive filtering.

Research Gap: Every IDS paper evaluates detection accuracy in isolation; every EKF paper assumes trustworthy sensor input. Prior work treats intrusion detection and state estimation as separate problems — this project closes the loop between them on embedded hardware.

# Chapter 5 & 20 — Core Theory & EKF Mathematical Derivation

State vector definition: x = [SoC, V_C1]^T. Battery state transitions follow 1RC equivalent circuit dynamics:

SoC(k+1) = SoC(k) - (eta * I(k) * dt) / Q_nom

V_C1(k+1) = V_C1(k) * exp(-dt / tau) + I(k) * R1 * (1 - exp(-dt / tau))

Measurement equation: V_pred = OCV(SoC) - V_C1 - I * R0. The innovation residual is y = V_meas - V_pred.

Kalman Gain: K = P * H^T / (H * P * H^T + R_eff).

Covariance Scaling Law (Patent Core): R_eff = R_base * exp(10 * S_anomaly). Under attack (S_anomaly -> 1.0), R_eff inflates by ~22,026x, driving K -> 0 and isolating the filter from malicious sensor measurements.

# Chapter 7 — Hardware Architecture, Voltage Scalability & Bill of Materials

Architecture Clarification (4S Bench Sub-Module vs 16S Traction Pack):

The physical prototype is constructed using a 4S 18650 modular sub-unit (14.8V nominal / 16.8V max) managed by a TI BQ76920 AFE IC. Full 16S (57.6V nominal / 67.2V max) or 100S EV traction packs are constructed by daisy-chaining 4x sub-modules over isolated SPI/CAN communication links, maintaining identical security and EKF algorithms per sub-unit.

# Chapter 8 — High-Side Solid-State Relay (SSR) Automated Disconnect

While low-side passive balancing MOSFETs (IRLML2502, 47 Ohm bleed) handle intra-cell voltage equalization, physical emergency disconnect of a compromised or hazardous pack requires a high-side disconnect switch.

When the Layer 3/5 ML Anomaly Engine computes score S > 0.90, GPIO 17 on the ESP32 BMS Master drives an optocoupler-isolated P-Channel Power MOSFET / SSR gate driver HIGH. This physically isolates the battery pack within < 1.2 ms, cutting load current to 0A without crashing the microcontroller or EKF state estimation.

# Chapter 9 — Layer 3 UDS (ISO 14229) Protocol Security & Secure Boot

The Layer 3 IDS rule engine extends coverage to Unified Diagnostic Services (UDS over ISO 14229 / OBD-II) on CAN IDs 0x7E0 (Tester) and 0x7E8 (ECU). The engine inspects Service Identifiers (SIDs) and intercepts unauthorized SecurityAccess (0x27) seed attempts or TesterPresent (0x3E) session keep-alives.

Firmware Integrity & Anti-Tampering: Production ESP32 deployments enable hardware Secure Boot V2 (RSA-3072 signature validation) and AES-256 eFuse Flash Encryption to prevent UART/JTAG binary extraction, bootloader manipulation, or unauthorized firmware flashing.

| Component Category | Baseline Choice | Low-Cost Optimized Choice | Cost Savings | Optimized Price |
| --- | --- | --- | --- | --- |
| Microcontrollers | 3x ESP32 Dev Boards (Rs. 1,140) | 1x ESP32-S3 / 2x Bare ICs + Laptop SLCAN | Rs. 380 - 450 | Rs. 690 |
| CAN Transceivers | 3x SN65HVD230 Modules (Rs. 360) | 2x VP230 / TJA1051 ICs + Manual Resistors | Rs. 120 | Rs. 240 |
| Battery Pack | 16x New 18650 Cells (Rs. 1,050) | 4S1P Reclaimed High-Drain 18650 Pack | Rs. 600 | Rs. 450 |
| Displays | 0.96" SSD1306 OLED (Rs. 90) | Omit OLED; Stream to Node-RED/Grafana/Serial | Rs. 90 | Rs. 0 |
| AFE & Sensors | BQ76920 + Shunt + NTC | BQ76920 Breakout Board + Shunt | Rs. 0 | Rs. 800 |
| High-Side SSR Cutoff | Standard Relay (Rs. 120) | High-Side SSR / Optocoupler P-FET Switch | Rs. 30 | Rs. 90 |
| Optimized Setup Total | Rs. 3,600 - 4,750 | Target Low-Cost Hardware Setup | Rs. 1,220 - 1,290 | Rs. 2,100 - 2,400 |

Table 7.1 — Low-Cost Optimized Bill of Materials (Target Rs. 2,100 – Rs. 2,400 per Prototype Setup)

# Chapter 22 & 23 — Firmware & Python Code Listings

1) BMS Master Firmware (bms_master.ino):

// Core 0 (Security): TWAI CAN RX -> UDS Inspector (0x7E0/0x27/0x3E) -> Random Forest -> FreeRTOS Queue

// Core 1 (Control): BQ76920 I2C -> EKF Predict/Update -> R_eff scaling -> SSR Cutoff (GPIO 17) -> Balancing

2) Attacker Firmware (attacker_node.ino):

// Modes: 1 -> DoS, 2 -> Spoof, 3 -> Replay, 4 -> Fuzz, 5 -> Mixed (1-7), 6 -> UDS Hijack, 7 -> Emergency SSR Test

3) Python ML Pipeline (train_ids.py & run_all_simulations.py):

# Feature engineering: InterArrival_ms, msg_freq, id_variance, entropy

# Model: RandomForestClassifier(n_estimators=20, max_depth=8)

# Export: m2cgen.export_to_c(model) -> ids_model.h