Cyber-Hardened Battery Management System for Electric Vehicles Using Edge ML Intrusion Detection and Adaptive EKF Covariance Scaling over CAN Bus

Department of Electrical & Electronics Engineering
Galgotias College of Engineering and Technology, Greater Noida, Uttar Pradesh, India

Abstract—Modern Electric Vehicles (EVs) rely on Controller Area Network (CAN, ISO 11898) broadcast telemetry for battery state estimation. Due to lack of message authentication, CAN networks are vulnerable to Denial-of-Service (DoS) and parameter spoofing attacks. In conventional Extended Kalman Filter (EKF) battery state estimators, corrupted telemetry causes severe divergence, yielding State-of-Charge (SoC) errors exceeding 18%. This paper presents a Cyber-Hardened BMS deployed on a dual-core ESP32 microcontroller (240 MHz) interfaced with a TI BQ76920 Analog Front-End. Core 0 executes a Random Forest Intrusion Detection System (IDS) compiled to native C++ via m2cgen, achieving 98.1% attack detection accuracy with <0.35 ms latency and zero GPU dependency. Core 1 executes an adaptive EKF where measurement noise covariance R_eff is exponentially scaled as R_eff = R_base × exp(λ × S_anomaly). Under attack (S_anomaly → 1), Kalman Gain K approaches zero, isolating state estimation from corrupted sensor data. Experimental validation confirms that under sustained DoS and spoofing, SoC estimation error remains below 1.4% (versus >18.4% in unprotected baselines) at a total hardware cost under ₹3,500 ($42 USD).

Keywords—Battery Management System, CAN Bus Security, Extended Kalman Filter, Random Forest, Edge ML, FreeRTOS, ESP32, ISO 11898, ISO/SAE 21434.

# I. INTRODUCTION

Electric Vehicle (EV) adoption relies heavily on the safety, longevity, and operational accuracy of Battery Management Systems (BMS). The BMS performs cell voltage monitoring, passive/active cell balancing, temperature tracking, and state estimation including State-of-Charge (SoC) and State-of-Health (SoH). Communication between the BMS, Motor Controller, and Vehicle Control Unit (VCU) relies on the Controller Area Network (CAN) protocol (ISO 11898).

However, CAN 2.0A/B lacks message authentication and sender validation. An attacker accessing the CAN bus via physical OBD-II diagnostic ports or wireless telematics units can flood the bus with dominant-identifier (0x000) frames or spoof telemetry payload bytes. When a traditional EKF state estimator receives corrupted voltage frames, the innovation residual forces state estimates to diverge, leading to dangerous over-discharge or erroneous cell balancing.

# II. PROPOSED SYSTEM ARCHITECTURE

The system architecture leverages a dual-core Xtensa LX6 processor (ESP32) running FreeRTOS. Core 0 handles CAN security classification, while Core 1 executes BQ76920 AFE acquisition, EKF state estimation, cell balancing, and OLED/SD logging.

1) Feature Extraction Engine: Telemetry inter-arrival time (Δt), rolling frame frequency (f), Data Length Code (DLC), and arbitration ID clusters are computed over a 100 ms sliding window.

2) Machine Learning IDS: A Random Forest model (10 decision trees, maximum depth 5) trained on 20,000 CAN frames is converted into pure C++ conditional IF/ELSE structures using m2cgen. This eliminates runtime heap allocation, requires ZERO GPU overhead, and reduces inference latency to <0.35 ms.

# III. MATHEMATICAL FORMULATION & ADAPTIVE EKF

The battery cell dynamics are modeled using a 1RC Equivalent Circuit Model (ECM):

V_t(k) = V_oc(SoC(k)) - I(k) \cdot R_0 - V_{C1}(k)    (1)

where V_oc(SoC) is open-circuit voltage, R_0 is ohmic resistance, and V_C1 is polarisation voltage. The state vector x_k = [SoC_k, V_{C1,k}]^T is propagated via Coulomb counting and RC dynamics:

x_{k|k-1} = f(x_{k-1}, I_{k-1})    (2)

P_{k|k-1} = A_{k-1} P_{k-1|k-1} A_{k-1}^T + Q    (3)

To harden the filter against malicious telemetry, the measurement noise covariance R_eff is dynamically modulated by the ML anomaly score S_anomaly ∈ [0.0, 1.0]:

R_{eff,k} = R_{base} \cdot e^{\lambda \cdot S_{anomaly,k}}    (4)

The Kalman Gain K_k and state update x_{k|k} are evaluated as:

K_k = P_{k|k-1} H_k^T \left( H_k P_{k|k-1} H_k^T + R_{eff,k} \right)^{-1}    (5)

x_{k|k} = x_{k|k-1} + K_k \left( y_k - h(x_{k|k-1}) \right)    (6)

Under cyber-attack (S_anomaly → 1.0), R_eff inflates exponentially by 22,026×, forcing K_k → 0. In Equation (6), K_k · y_k approaches zero, suppressing corrupted sensor input y_k and forcing x_{k|k} ≈ x_{k|k-1} (model-only state propagation).

# IV. EXPANDED EXPERIMENTAL RESULTS & BENCHMARKS

A. IDS Classification Performance
Evaluation on 20,000 CAN telemetry frames (16,000 normal, 4,000 attack) yields the confusion matrix detailed in Table I.

| Actual \ Predicted | Normal (Pred) | Attack (Pred) | Total |
| --- | --- | --- | --- |
| Normal (Actual) | 15,842 (TN) | 158 (FP) | 16,000 |
| Attack (Actual) | 229 (FN) | 3,771 (TP) | 4,000 |
| Metrics | Accuracy: 98.1% | Precision: 95.98% | Recall: 94.28% |

Table I: Random Forest IDS Confusion Matrix (20,000 Test Frames)

B. ROC Curve & Latency Analysis
The classifier achieves an Area Under Curve (ROC-AUC) of 0.994. Thanks to m2cgen C++ IF/ELSE compilation, execution latency on ESP32 Core 0 averages 0.24 ms with a 99th percentile of 0.33 ms (well below the 2.0 ms CAN frame budget at 500 kbps).

C. SoC Estimation Performance Under Attack
During a 20-second simulated DoS flooding and voltage spoofing attack, baseline unprotected EKF deviates significantly, incurring an SoC error of 18.4%. The proposed Cyber-Hardened EKF limits maximum SoC error to 1.4% (Figure 2 benchmarking). Post-attack re-convergence occurs in under 300 ms (3 EKF cycles).

# V. CONCLUSION & FUTURE SCOPE

A cyber-hardened BMS combining edge ML classification with adaptive EKF covariance scaling was developed and validated. Future scope includes upgrading physical transceivers to CAN-FD via an SPI-attached MCP2518FD controller.